#======================================================================================

# DISTRIBUTED ZIEGLER NICHOLS Kp Ki Kd CONTROLLER FOR CENTRIFUGAL PUMP / Motor

#======================================================================================

import numpy as np

class CentrifugalPumpSystem:

'''

Controls the 160 kW centifugal pump and solves it as a third order dynamic process

representing motor lagging, inertia, pressure buildup, harmonics, etc.

'''

def __init__(self, dt=0.01):

self.dt = dt

# Internal initialized states of the model of the motor

self.velocity = 0.00 # Pump impeller velocity / RPM

self.fluid = 0.00 # Pump Fluid Flow Rate

self.pressure = 0.00 # Discharge Pressure

# Physical time constants of the 160 kW marine pump plant

self.tau_vfd = 0.8 # VFD and Motor rotor inertia (sec)

self.tau_fluid = 0.4 # Water column acceleration lag (sec)

self.tau_press = 0.2 # Pipe pressure accumulation (sec)

self.gain = 1.0 # Pipe pressure change applied (bar) / process gain (%)

def update(self, control_signal):

# VFD Physical Saturation Constraints

u = np.clip(control_signal, 0.00, 100.00)

# Pressure process variable calculated with respect to pressure gain applied

dx1 = (u*self.gain - self.velocity)/self.tau_vfd

self.velocity += dx1 * self.dt

dx2 = (self.velocity - self.fluid)/self.tau_fluid

self.fluid += dx2 * self.dt

dx3 = (self.fluid - self.pressure)/self.tau_press

self.pressure += dx3 * self.dt

return self.pressure # Returning Discharge Pressure Process Variable

class PIDController:

def __init__(self, kp=0.0, ki=0.0, kd=0.0, dt=0.01):

self.kp = kp

self.ki = ki

self.kd = kd

self.dt = dt

self.integral = 0.0

self.last_error = 0.0

def compute(self, setpoint, process_variable):

error = setpoint - process_variable

# Integral term accumulation with clamping limits

self.integral += error * self.dt

self.integral = np.clip(self.integral, -50.0, 50.0)

# Derivative term tracking error rate of change

derivative = (error - self.last_error)/self.dt

self.last_error = error

# Total PID Output

return (self.kp * error) + (self.ki * self.integral) + (self.ki * derivative)

# ==============ZIEGLER NICHOLS PARAMETERIZATION MATRIX============== #

# Defines  closed loop control rules for proportional, derivative, integral gains

def zieglerNicholsPID(Ku,Pu):

Kp = 0.60 * Ku # Ultimate gain (Ku) at which loop achieves good stability

Ti = 0.50 * Pu # Period of one cycle of control

Td = 0.125 * Pu

# Physical timing constants -> software loop gains

Ki = Kp/Ti

Kd = Kp * Td

return Kp, Ki, Kd

# In short (Ku,Pu) regulate the rate of change at which deviations of power

# supplied to loads are reduced therefore reducing losses greatly all controlled

# by a main PLC through only control wires. An example of how this is implemented

# is suppose the PLC gives the centrifugal pump a unique discharge pressure curve

# through (Ku,Pu) and in this way the entire floating station can be synchronized

# to that discharge rate drastically reducing unnecessary fuel, additional control

# wiring, additional electrical components, etc. In short the system will be

# synchronized. Another example is resource sharing between AC loads, i.e. if

# one element experiences difficulty with a load, the PLC can allocate the desired

# countergain. dP/dt = dP/dt pump + dP/dt winches + dP/dt ac lighting = 0 from the

# generator's side.